# SILICON DREAMS

Study Guide

MODULE 3  ·  WEEK 3  ·  CM-HW-101-M3

SG-M3-02    pairs with TB-M3-02

# Top-Level Integration Lab

Wire elevator, arbiter, clock_gate, and axiom_shim into a single tt_um compliant top.

## Learning objectives

- Translate the tt_um pinout contract into named-port bindings inside top.v.

- Use valid/ready handshakes between elevator and arbiter without deadlocking either side.

- Correctly partition reset domains — elevator_rst_n, arbiter_rst_n, axiom_rst_n — and understand why they must NOT be tied together.

- Run the integration smoke test and read its FSM trace to confirm all four sub-modules wake up in the right order.

## Prep checklist

- SG-M3-01 complete, first LibreLane run finished (even if DRC failed).

- You have re-read TB-M3-02 — especially the pinout allocation table.

- Your M2 elevator.v compiles standalone (run `make lint` in src/).

## Step 1 · Read the pinout contract

ChipFoundry shuttle tiles expose exactly 8 input pins, 8 output pins, and 8 bidirectional pins. They are named `ui_in`, `uo_out`, and `uio`. Every top.v in the cohort must match this contract — the pad ring is fixed.

module tt_um_silicon_dreams (

input  wire [7:0] ui_in,    // dedicated inputs

output wire [7:0] uo_out,   // dedicated outputs

input  wire [7:0] uio_in,   // bidirectional inputs

output wire [7:0] uio_out,  // bidirectional outputs

output wire [7:0] uio_oe,   // bidirectional output-enable

input  wire       ena,      // always 1 when the design is selected

input  wire       clk,

input  wire       rst_n     // active-LOW reset (important!)

);

E8873C

rst_n is ACTIVE-LOW. Your hardened M2 elevator already handles this, but arbiter, clock_gate, and axiom_shim may need a polarity flip before being fed their own reset signals.

## Step 2 · Fill in the M3 pinout allocation

From TB-M3-02 Table 2.1:

| ui_in[0] | request_strobe → elevator |
| --- | --- |
| ui_in[4:1] | requested_floor[3:0] → elevator |
| ui_in[5] | priority_override_req → arbiter |
| ui_in[6] | axiom_enable → axiom_shim |
| ui_in[7] | debug_probe_select |
| uo_out[3:0] | current_floor[3:0] ← elevator |
| uo_out[6:4] | state[2:0] ← elevator |
| uo_out[7] | door_open ← elevator |
| uio[0] | fault_inject_enable (input; held in M3 too) |
| uio[1] | axiom_misbehaviour_led (output) |
| uio[2] | arbiter_grant_elevator (output, debug) |
| uio[3] | arbiter_grant_axiom (output, debug) |
| uio[7:4] | reserved / error + clock-gate status |

## Step 3 · Map ports, not wires

Always use NAMED port connections. Positional connections break horribly during integration and the course lint will reject them.

// GOOD

elevator u_elevator (

.clk             (clk),

.rst_n           (elevator_rst_n),

.request_strobe  (request_strobe),

.requested_floor (requested_floor),

.fault_inject_en (fault_inject_enable),

.current_floor   (current_floor),

.state           (elevator_state),

.door_open       (door_open),

.error_led       (elevator_error_led)

);

// BAD

elevator u_elevator (clk, elevator_rst_n, ui_in[0], ...);

## Step 4 · Reset domain partitioning

Every sub-module gets its OWN reset signal derived from rst_n. Do NOT tie them together. The reasons:

- elevator_rst_n is held low for 4 cycles after rst_n deasserts — this lets the elevator state machine arrive in IDLE deterministically.

- arbiter_rst_n is held low for 6 cycles — the arbiter has a 3-deep priority queue that needs an extra two cycles to flush.

- axiom_rst_n is INVERTED (active-HIGH for axiom_blackbox). See TB-M3-05 defensive rule #1.

// reset_synchroniser.v pattern — used three times in top.v

reset_synchroniser #(.HOLD_CYCLES(4)) u_rst_elev (

.clk   (clk),

.rst_n (rst_n),

.rst_n_sync (elevator_rst_n)

);

reset_synchroniser #(.HOLD_CYCLES(6)) u_rst_arb  (.clk(clk), .rst_n(rst_n), .rst_n_sync(arbiter_rst_n));

assign axiom_rst = ~axiom_rst_n_sync;  // polarity flip at the boundary

## Step 5 · Valid/ready handshake between elevator and arbiter

The elevator emits a floor-request packet whenever a request_strobe lands. The arbiter must acknowledge before the elevator can emit another.

// Inside top.v

wire [3:0] req_payload;

wire       req_valid;

wire       req_ready;

elevator_req_port u_req_port (

.clk         (clk),

.rst_n       (elevator_rst_n),

.strobe      (request_strobe),

.floor       (requested_floor),

.req_payload (req_payload),

.req_valid   (req_valid),

.req_ready   (req_ready)

);

arbiter u_arbiter (

.clk             (clk),

.rst_n           (arbiter_rst_n),

.priority_req    (priority_override),

.elev_req_valid  (req_valid),

.elev_req_ready  (req_ready),

.elev_req_payload(req_payload),

.grant_elev      (grant_elev),

.grant_axiom     (grant_axiom),

.queue_nonempty  (arbiter_queue_nonempty)

);

6B2E8F

Rule: `valid` MUST NOT depend combinationally on `ready`. If it does, the arbiter cannot assert ready without triggering another valid — infinite handshake loop. Register the valid signal on the elevator side.

## Step 6 · Run the integration smoke test

cd test/top

make MODULE=test_top_smoke

# Expected output tail:

# == TOP INTEGRATION SMOKE =============================

#   reset_release ............................. PASS

#   elevator_wakes_in_IDLE .................... PASS

#   arbiter_grants_elevator_by_default ........ PASS

#   priority_override_grants_axiom ............ PASS

#   axiom_shim_clamps_misbehaviour ............ PASS

#   clock_gate_disables_on_idle ............... PASS

# == SUMMARY ==========================================

#   8 pass, 0 fail, 0 skip

If reset_release fails, you have a reset-domain bug. If elevator_wakes_in_IDLE fails, you have an M2 regression — go back. If arbiter_grants_elevator_by_default fails, you have a priority logic bug (preview of SG-M3-03).

## Troubleshooting

| `Error: ui_in unused` | You forgot to wire ui_in[5] (priority_override_req). Lint will flag this. |
| --- | --- |
| `Error: uio_oe missing` | Every uio pin you drive must set its uio_oe bit. See pinout doc. |
| Smoke test hangs | Likely a handshake deadlock. Add `$monitor(req_valid, req_ready)` and re-run — look for both stuck at X or at 1. |
| `axiom_blackbox not found` during synth | axiom_blackbox.v is a BEHAVIOURAL stub only. It is bound by the grader at P&R time. Do not try to flatten it. |

## Reflection

- Draw top.v as a box diagram with five boxes (elevator, arbiter, clock_gate, axiom_shim, reset_syncs) and label every connecting wire. Save this diagram as `docs/top-block-diagram.png`.

- Confirm all six smoke tests pass. If even one fails, do NOT proceed to SG-M3-03.