Silicon Dreams · CM-HW-101-M3 · SG-M3-02 · ChipMango × ChipFoundry

SILICON DREAMS

Study Guide

MODULE 3 · WEEK 3 · CM-HW-101-M3

SG-M3-02 pairs with TB-M3-02

Top-Level Integration Lab

Wire elevator, arbiter, clock_gate, and axiom_shim into a single tt_um compliant  top.

Learning objectives

• Translate the tt_um pinout contract into named-port bindings inside top.v.  • Use valid/ready handshakes between elevator and arbiter without deadlocking either side.

• Correctly partition reset domains — elevator_rst_n, arbiter_rst_n, axiom_rst_n — and  understand why they must NOT be tied together.

• Run the integration smoke test and read its FSM trace to confirm all four sub-modules wake up  in the right order.

Prep checklist

☐ SG-M3-01 complete, first LibreLane run finished (even if DRC failed).

☐ You have re-read TB-M3-02 — especially the pinout allocation table.

☐ Your M2 elevator.v compiles standalone (run `make lint` in src/).

Step 1 · Read the pinout contract

ChipFoundry shuttle tiles expose exactly 8 input pins, 8 output pins, and 8 bidirectional pins. They are  named `ui_in`, `uo_out`, and `uio`. Every top.v in the cohort must match this contract — the pad ring is  fixed.

module tt_um_silicon_dreams (

input wire [7:0] ui_in, // dedicated inputs

output wire [7:0] uo_out, // dedicated outputs

input wire [7:0] uio_in, // bidirectional inputs

output wire [7:0] uio_out, // bidirectional outputs

output wire [7:0] uio_oe, // bidirectional output-enable

chipmango.io · Silicon Dreams · Module 3 · SG-M3-02 · p.1

Silicon Dreams · CM-HW-101-M3 · SG-M3-02 · ChipMango × ChipFoundry

input wire ena, // always 1 when the design is selected   input wire clk,

input wire rst_n // active-LOW reset (important!)  );

E8873C

rst_n is ACTIVE-LOW. Your hardened M2 elevator already handles this, but arbiter, clock_gate, and  axiom_shim may need a polarity flip before being fed their own reset signals.

Step 2 · Fill in the M3 pinout allocation

From TB-M3-02 Table 2.1:

ui_in[0] request_strobe → elevator

ui_in[4:1] requested_floor[3:0] → elevator

ui_in[5] priority_override_req → arbiter

ui_in[6] axiom_enable → axiom_shim

ui_in[7] debug_probe_select

uo_out[3:0] current_floor[3:0] ← elevator

uo_out[6:4] state[2:0] ← elevator

uo_out[7] door_open ← elevator

uio[0] fault_inject_enable (input; held in M3 too)

uio[1] axiom_misbehaviour_led (output)

uio[2] arbiter_grant_elevator (output, debug)

uio[3] arbiter_grant_axiom (output, debug)

uio[7:4]

reserved / error + clock-gate status

Step 3 · Map ports, not wires

Always use NAMED port connections. Positional connections break horribly during integration and the  course lint will reject them.

// GOOD

elevator u_elevator (

.clk (clk),

.rst_n (elevator_rst_n),

.request_strobe (ui_in[0]),

.requested_floor(ui_in[4:1]),

.current_floor (current_floor),

.state (elevator_state),

.door_open (door_open),

chipmango.io · Silicon Dreams · Module 3 · SG-M3-02 · p.2

Silicon Dreams · CM-HW-101-M3 · SG-M3-02 · ChipMango × ChipFoundry

.error_led (elevator_error)

);

// BAD

elevator u_elevator (clk, elevator_rst_n, ui_in[0], ...);

Step 4 · Reset domain partitioning

Every sub-module gets its OWN reset signal derived from rst_n. Do NOT tie them together. The reasons:

• elevator_rst_n is held low for 4 cycles after rst_n deasserts — this lets the elevator state  machine arrive in IDLE deterministically.

• arbiter_rst_n is held low for 6 cycles — the arbiter has a 3-deep priority queue that needs an  extra two cycles to flush.

• axiom_rst_n is INVERTED (active-HIGH for axiom_blackbox). See TB-M3-05 defensive rule #1.  // reset_synchroniser.v pattern — used three times in top.v

reset_synchroniser #(.HOLD_CYCLES(4)) u_rst_elev (

.clk (clk),

.rst_n (rst_n),

.rst_n_sync (elevator_rst_n)

);

reset_synchroniser #(.HOLD_CYCLES(6)) u_rst_arb (.clk(clk), .rst_n(rst_n),  .rst_n_sync(arbiter_rst_n));

assign axiom_rst = ~axiom_rst_n_sync; // polarity flip at the boundary

Step 5 · Valid/ready handshake between elevator and arbiter

The elevator emits a floor-request packet whenever a request_strobe lands. The arbiter must  acknowledge before the elevator can emit another.

// Inside top.v

wire [3:0] req_payload;

wire req_valid;

wire req_ready;

elevator_req_port u_req_port (

.clk (clk),

.rst_n (elevator_rst_n),

.strobe (ui_in[0]),

.floor (ui_in[4:1]),

.req_payload (req_payload),

.req_valid (req_valid),

.req_ready (req_ready)

chipmango.io · Silicon Dreams · Module 3 · SG-M3-02 · p.3

Silicon Dreams · CM-HW-101-M3 · SG-M3-02 · ChipMango × ChipFoundry

);

arbiter u_arbiter (

.clk (clk),

.rst_n (arbiter_rst_n),

.priority_req (ui_in[5]),

.elev_req_valid (req_valid),

.elev_req_ready (req_ready),

.elev_req_payload(req_payload),

.grant_elev (uio_out[2]),

.grant_axiom (uio_out[3])

);

6B2E8F

Rule: `valid` MUST NOT depend combinationally on `ready`. If it does, the arbiter cannot assert ready  without triggering another valid — infinite handshake loop. Register the valid signal on the elevator  side.

Step 6 · Run the integration smoke test

cd test/top

make MODULE=test_top_smoke

# Expected output tail:

# == TOP INTEGRATION SMOKE =============================

# reset_release ............................. PASS

# elevator_wakes_in_IDLE .................... PASS

# arbiter_grants_elevator_by_default ........ PASS

# priority_override_grants_axiom ............ PASS

# axiom_shim_clamps_misbehaviour ............ PASS

# clock_gate_disables_on_idle ............... PASS

# == SUMMARY ==========================================

# 6 pass, 0 fail, 0 skip

If reset_release fails, you have a reset-domain bug. If elevator_wakes_in_IDLE fails, you have an M2  regression — go back. If arbiter_grants_elevator_by_default fails, you have a priority logic bug (preview  of SG-M3-03).

Troubleshooting

`Error: ui_in unused` You forgot to wire ui_in[5] (priority_override_req). Lint will flag  this.

`Error: uio_oe missing`

Every uio pin you drive must set its uio_oe bit. See pinout doc.

chipmango.io · Silicon Dreams · Module 3 · SG-M3-02 · p.4

Silicon Dreams · CM-HW-101-M3 · SG-M3-02 · ChipMango × ChipFoundry

Smoke test hangs Likely a handshake deadlock. Add `$monitor(req_valid,  req_ready)` and re-run — look for both stuck at X or at 1.

`axiom_blackbox not found` during synth Reflection

axiom_blackbox.v is a BEHAVIOURAL stub only. It is bound by the grader at P&R time. Do not try to flatten it.

• Draw top.v as a box diagram with five boxes (elevator, arbiter, clock_gate, axiom_shim,  reset_syncs) and label every connecting wire. Save this diagram as `docs/top-block diagram.png`.

• Confirm all six smoke tests pass. If even one fails, do NOT proceed to SG-M3-03.

chipmango.io · Silicon Dreams · Module 3 · SG-M3-02 · p.5