# SILICON DREAMS

Study Guide

MODULE 3  ·  WEEK 3  ·  CM-HW-101-M3

SG-M3-03    pairs with TB-M3-03

# The Arbiter: Round-Robin with Priority Override

Find and fix the starter arbiter's fairness bug; prove correctness with a cocotb test.

## Learning objectives

- Read the starter arbiter.v and identify the one-line bug that violates round-robin fairness.

- Understand the priority-override contract — when it must preempt and when it must NOT starve the elevator.

- Write a cocotb test that forces 1000 back-to-back requests and asserts fairness holds within 2%.

- Earn the 50 XP bonus for an alternate fix that meets timing at 100 MHz on the first try.

## Prep checklist

- SG-M3-02 complete, integration smoke green.

- TB-M3-03 read, especially the pseudocode in §3.2.

- You understand the contract: priority_override_req, when 1, grants axiom ON THE NEXT CYCLE. When 0, arbiter round-robins between elevator and axiom.

## Step 1 · Read the starter RTL

// src/arbiter.v (starter, buggy)

module arbiter (

input  wire       clk,

input  wire       rst_n,

input  wire       priority_req,

input  wire       elev_req_valid,

output wire       elev_req_ready,

input  wire [3:0] elev_req_payload,

output reg        grant_elev,

output reg        grant_axiom,

output wire       queue_nonempty

);

reg last_granted_axiom;

reg pending_request;

always @(posedge clk or negedge rst_n) begin

if (!rst_n) begin

grant_elev         <= 1'b0;

grant_axiom        <= 1'b0;

last_granted_axiom <= 1'b0;

pending_request    <= 1'b0;

end else begin

grant_elev  <= 1'b0;

grant_axiom <= 1'b0;

// Track pending requests

if (elev_req_valid && !grant_elev) begin

pending_request <= 1'b1;

end else if (grant_elev) begin

pending_request <= 1'b0;

end

// Priority mode: AXIOM gets priority, but alternates to be fair

if (priority_req) begin

if (last_granted_axiom) begin

grant_elev         <= 1'b1;

grant_axiom        <= 1'b0;

last_granted_axiom <= 1'b0;

end else begin

grant_elev         <= 1'b0;

grant_axiom        <= 1'b1;

last_granted_axiom <= 1'b1;

end

end

// Round-robin mode

// BUG LIVES HERE — see SG-M3-03 §Step 2

else if (elev_req_valid || pending_request) begin

if (last_granted_axiom) begin

grant_elev         <= 1'b1;

grant_axiom        <= 1'b0;

last_granted_axiom <= 1'b0;

end else begin

grant_elev         <= 1'b0;

grant_axiom        <= 1'b1;

last_granted_axiom <= 1'b10;

end

end

end

end

assign elev_req_ready = grant_elev;

assign queue_nonempty = elev_req_valid || pending_request;

endmodule

## Step 2 · Find the fairness violation

The priority path writes to grant_elev and grant_axiom but does NOT update last_granted_axiom. Consequence: an adversary that wiggles priority_req every cycle causes the arbiter to grant axiom every cycle forever — the elevator starves.

This is a classic priority-inversion bug. The fairness contract says: priority_req may PREEMPT, but must also COUNT as an axiom grant so that the next non-priority cycle round-robins to elevator.

## Step 3 · Write the fairness test

Add to `test/arbiter/test_fairness.py`:

"""Fairness test for arbiter.v

Tests that priority_req does not starve the elevator.

"""

import cocotb

from cocotb.clock import Clock

from cocotb.triggers import RisingEdge, Timer

CLK_PERIOD_NS = 10

RESET_CYCLES  = 8

async def start_clock(dut):

cocotb.start_soon(Clock(dut.clk, CLK_PERIOD_NS, units="ns").start())

async def nominal_reset(dut):

dut.rst_n.value = 0

for _ in range(RESET_CYCLES):

await RisingEdge(dut.clk)

dut.rst_n.value = 1

for _ in range(RESET_CYCLES):

await RisingEdge(dut.clk)

@cocotb.test()

async def test_priority_does_not_starve_elevator(dut):

"""Priority-req adversary: hold priority_req for most cycles;

elevator must still receive at least one grant within the 2% fairness window."""

dut._log.info("=== Starting fairness test ===")

await start_clock(dut)

await nominal_reset(dut)

# Start with priority high

dut.priority_req.value   = 1

dut.elev_req_valid.value = 1

dut.elev_req_payload.value = 0x3

grants_elev = 0

grants_axiom = 0

dut._log.info("Running 1000 cycles with priority_req=1, dropping every 10 cycles...")

for i in range(1000):

# Drop priority every 10 cycles to let elevator through

if i % 10 == 5:

dut.priority_req.value = 0

else:

dut.priority_req.value = 1

await RisingEdge(dut.clk)

grants_elev  += int(dut.grant_elev.value)

grants_axiom += int(dut.grant_axiom.value)

if (i + 1) % 100 == 0:

dut._log.info(f"Cycle {i+1}: elev={grants_elev}, axiom={grants_axiom}")

total = grants_elev + grants_axiom

elev_share = grants_elev / total if total else 0

dut._log.info(f"=== Results ===")

dut._log.info(f"  grants_elev  = {grants_elev}")

dut._log.info(f"  grants_axiom = {grants_axiom}")

dut._log.info(f"  total        = {total}")

dut._log.info(f"  elev_share   = {elev_share:.3f}")

dut._log.info(f"  required     >= 0.020 (2%)")

assert elev_share >= 0.02, (

f"elevator starved: elev={grants_elev} axiom={grants_axiom} "

f"share={elev_share:.3f} < 0.02"

)

dut._log.info(">>> Fairness test PASSED! <<<")

@cocotb.test()

async def test_round_robin_works(dut):

"""With priority_req=0, elevator and AXIOM should alternate."""

dut._log.info("=== Starting round-robin test ===")

await start_clock(dut)

await nominal_reset(dut)

dut.priority_req.value   = 0

dut.elev_req_valid.value = 1

dut.elev_req_payload.value = 0x3

grants_elev = 0

grants_axiom = 0

for i in range(100):

await RisingEdge(dut.clk)

grants_elev  += int(dut.grant_elev.value)

grants_axiom += int(dut.grant_axiom.value)

total = grants_elev + grants_axiom

elev_share = grants_elev / total if total else 0

dut._log.info(f"=== Results ===")

dut._log.info(f"  grants_elev  = {grants_elev}")

dut._log.info(f"  grants_axiom = {grants_axiom}")

dut._log.info(f"  elev_share   = {elev_share:.3f}")

assert 0.4 < elev_share < 0.6, (

f"Round-robin not working: elev_share={elev_share:.3f}, expected ~0.5"

)

dut._log.info(">>> Round-robin test PASSED! <<<")

Run it:

cd test/arbiter

make clean

make MODULE=test_fairness

## Step 4 · Apply the minimum fix

One line. In the priority branch:

else if (elev_req_valid || pending_request) begin

if (last_granted_axiom) begin

grant_elev         <= 1'b1;

grant_axiom        <= 1'b0;

last_granted_axiom <= 1'b0;

end else begin

grant_elev         <= 1'b0;

grant_axiom        <= 1'b1;

last_granted_axiom <= 1'b1;   ←—--- Fixed

end

Re-run the fairness test. It should now pass within 2% elevator share. If your share drops below 2% even with the fix, your reset path is not clearing last_cd test/arbiter granted_axiom — check the `if (!rst_n)` branch.

make clean

make MODULE=test_fairness

# Expected (with the bug):

#   test_priority_does_not_starve_elevator: PASS

Grants: elev=499, axiom=500

elev_share: 0.499 (≥ 0.020 requirement)

test_round_robin_works: PASSED

Grants: elev=50, axiom=50

elev_share: 0.500

## Step 5 · Bonus — 50 XP for a parameterised round-robin

The minimum fix covers 2-way arbitration. The bonus path generalises to N-way, readable in one line of Verilog for any N:

parameter N = 2;

reg [N-1:0] grant_onehot, req_onehot;

// rotate a one-hot pointer right, gate it by pending requests

wire [N-1:0] next_grant = rotate_right(grant_onehot) & req_onehot;

The bonus 50 XP is awarded for: parameterised arbiter that works at N=2 and N=4, fairness test holds, and clocked timing meets 100 MHz on the first LibreLane try. You do NOT need to instantiate N=4 in top.v — the grader will sweep it on a side testbench.

## Step 6 · Regenerate metrics

./scripts/run-librelane.sh --tag arbiter-fix

cat ./librelane/runs/$(ls -t ./librelane/runs/ | head -1)/55-openroad-stapostpnr/summary.rpt

Setup WNS (TT Corner):  5.4058 ns

Setup WNS (Overall):    3.9761 ns

Hold WNS (Overall):     0.1620 ns

Violations:             0

# WNS (worst negative slack) positive or zero at CLOCK_PERIOD=10ns

cat ./librelane/runs/$(ls -t ./librelane/runs/ | head -1)/55-openroad-stapostpnr/nom_tt_025C_1v80/ws.max.rpt

===========================================================================

Worst Slack (Setup)

============================================================================

nom_tt_025C_1v80: 5.405828382386134

## Troubleshooting

| Fairness test passes but timing fails | Your fix introduced a long combinational path. Register the next_grant. Don't collapse it into assign. |
| --- | --- |
| `last_granted_axiom` shown as X in waves | You missed it in the `if (!rst_n)` branch. Add `last_granted_axiom <= 1'b0;`. |
| elev_share > 0.98 | You may have broken priority_req — it's now being ignored. Re-check the priority branch still sets grant_axiom=1. |

## Reflection

- Which view helped you find the bug first — reading the code, reading the waveform, or reading the failing test output? Note it for your escape log.

- Write a one-sentence summary of the fairness contract in your own words. If you can't, re-read TB-M3-03 §3.2.