# SILICON DREAMS

Study Guide

MODULE 3  ·  WEEK 3  ·  CM-HW-101-M3

SG-M3-06    pairs with TB-M3-06

# LibreLane and DRC Fixes

Read a DRC report, classify the top five violation types, fix them in the correct layer of the flow.

## Learning objectives

- Navigate runs/<tag>/reports/signoff/drc.rpt and read Magic's error format.

- Classify every DRC error into one of five buckets: met1 spacing, antenna ratio, tap-cell spacing, power via enclosure, off-grid pins.

- Know which config knob or RTL change fixes each bucket — NOT all DRC errors are fixed the same way.

- Achieve zero DRC on the full design.

## Step 1 · Locate the DRC report

cd runs/<latest-tag>/reports/signoff

less drc.rpt

# Format:

# [CATEGORY] line: <error message>

# cell: <offending cell>

# box: ( xmin ymin xmax ymax )

# layer: met1 / met2 / tap / etc.

## Step 2 · The five buckets

| met1 spacing | Two metal-1 shapes violate minimum spacing. Usually caused by high PL_TARGET_DENSITY_PCT. Lower from 0.60 to 0.55, i In some versions of the tool, density is expressed as a decimal: 0.55 = 55%, 0.60 = 60% re-run. |
| --- | --- |
| antenna ratio | A long metal wire acts as an antenna during fab, can damage gates. Fix: insert antenna diodes (`DIODE_INSERTION_STRATEGY=3` in config.json) or break the net with a buffer. |
| tap cell spacing | sky130 requires a tap cell every 15 µm. If you see this, your floorplan has a gap. Set `FP_TAP_DISTANCE=14` to force denser tap insertion. |
| power via enclosure | The via from met1 to met4 in the power straps isn't fully enclosed by the surrounding metal. Fix: bump `FP_PDN_ENABLE_RAILS_POWER_GRID` and widen the strap — usually a floorplan config knob, NOT an RTL fix. |
| off-grid pins | Your pad-ring pin is not on the manufacturing grid. Check `pin_order.cfg` — x/y must be multiples of 0.005 µm. |

## Step 3 · Worked example — met1 spacing

Typical snippet of drc.rpt for this category:

[DRC-1] line 443: Metal1 to Metal1 spacing violation (min 0.14 µm)

cell: _24837_

box:  (12.345 34.567 12.485 34.707)

layer: met1

severity: ERROR

The fix:

# Edit librelane/config.json

"PL_TARGET_DENSITY_PCT": 0.55,   # was 0.60

"ROUTING_OPT_ITERS": 32,         # was 16 — more passes give router more room

# Re-run

./scripts/run-librelane.sh

# Diff the report line counts

wc -l runs/first-run/reports/signoff/drc.rpt \

runs/drc-fix-1/reports/signoff/drc.rpt

## Step 4 · Worked example — antenna

# config.json

"DIODE_INSERTION_STRATEGY": 3,

"GRT_REPAIR_ANTENNAS": 1

Strategy 3 inserts antenna diodes on every high-antenna-ratio net. If you're power-tight, try strategy 2 first — it is less invasive but won't fix all cases.

## Step 5 · Iterate

DRC fixing is iterative. Each pass:

- Read the top error category by count.

- Apply the canonical fix for that category (see table above).

- Re-run LibreLane with a new tag.

- Compare drc.rpt line count. Expect exponential decay.

Most designs in this cohort reach zero DRC in 3-4 iterations. If you're on iteration 6 and still have >50 errors, you likely have a structural floorplan problem — escalate in Discord #m3-drc.

## Step 6 · CI integration

`.github/workflows/drc.yml` POSTs every failing DRC report to the ChipFoundry grader. It does NOT fail the PR on DRC errors (those are noisy during iteration), but it DOES fail the final-sign-off tag `v1.0.0-final`.

# Tag the run when you believe it's clean

git tag v1.0.0-final

git push --tags

# CI picks up the tag and runs:

#   1. DRC must be 0 errors

#   2. LVS must be MATCH

#   3. Timing WNS must be >= 0 (worst corner)

# Any fail → tag is rejected, you must re-tag as v1.0.1-final etc.

## Troubleshooting

| DRC count oscillates between runs | LibreLane is non-deterministic in placement. Pin the random seed in config.json: "PL_RANDOM_INITIAL_PLACEMENT_SEED": 42 |
| --- | --- |
| Same error after fix | Your fix was in the wrong config key. Double-check the spelling — LibreLane doesn't error on unknown keys. |
| New category appears after fix | Normal. Fixing one category can shift placement and expose another. Keep iterating. |
| drc.rpt is empty but CI says DRC failed | Check drc.klayout.xml — KLayout runs a second DRC pass with different rules. Both must be clean. |

## Reflection

- Record your DRC error count per iteration. Plot it — the curve should be roughly geometric.

- Which bucket gave you the most trouble? That's the one to write up in your escape log. Expect 'met1 spacing' for most learners.