# SILICON DREAMS

Study Guide

MODULE 3  ·  WEEK 3  ·  CM-HW-101-M3

SG-M3-07    pairs with TB-M3-07

# LVS and Timing Closure

Diagnose the three canonical LVS fail patterns; close timing on the worst corner with two of four techniques.

## Learning objectives

- Read an LVS report and classify the fail into one of three patterns: port name mismatch, net connectivity mismatch, cell count mismatch.

- Understand setup, hold, and why both corners matter.

- Apply two of four timing closure techniques: tighten SDC, re-structure RTL critical path, upsize flop drive strength, or bump CLOCK_PERIOD.

- Achieve worst-corner WNS ≥ 0 and hold slack ≥ 0.

## Step 1 · Read the LVS report

cd runs/<latest-tag>/reports/signoff

less lvs.rpt

# Top of a healthy report looks like:

#   Circuit 1: layout

#   Circuit 2: schematic

#   Circuits match: MATCH

#

# An unhealthy one:

#   Port mismatch on "axiom_misbehaviour":

#     layout has "axiom_misbehaviour" as uio_out[1]

#     schematic has "axiom_misbeh_out" as uio_out[1]

## Step 2 · Pattern A — Port name mismatch

Symptom: LVS fails with 'Port mismatch on <name>'. Cause: you renamed a Verilog port but the pad ring / pin_order config still uses the old name.

# Fix: update librelane/pin_order.cfg

# BEFORE:

#   uio_out[1] axiom_misbeh_out

# AFTER:

#   uio_out[1] axiom_misbehaviour

# Re-run

./scripts/run-librelane.sh

#check thr lvs-report

runs/reports/signoff/sta-worst.rpt

## Step 3 · Pattern B — Net connectivity mismatch

Symptom: 'Net <N> has different topology in layout vs schematic'. Cause: a `wire` was left dangling because a port on a submodule is not hooked up in top.v.

# Find it quickly:

grep -n "assign" src/top.v | grep uio_out

# Look for missing assigns or conflicting drivers.

# Classic case: you declared uio_out[3] as a debug output but never drove it.

# Fix in top.v:

assign uio_out[3] = arbiter_grant_axiom;

assign uio_oe[3]  = 1'b1;

## Step 4 · Pattern C — Cell count mismatch

Symptom: 'Circuit 1 has 1247 cells; Circuit 2 has 1245 cells'. Cause: synthesis inserted a cell (typically a buffer for fanout) that the schematic extraction didn't see, OR a constant has been optimised away in one but not the other.

Fix: usually a re-run after fixing Pattern A or B resolves it. If persistent, the issue is in the liberty file or a missing model for a non-standard cell. Check `logs/signoff/magic-extract.log` for unresolved references.

## Step 5 · Read the timing report

less runs/<tag>/reports/signoff/sta-worst.rpt

# Key fields:

# Startpoint: <flop_X>

# Endpoint:   <flop_Y>

# Path Type:  max (setup) or min (hold)

# required time:   10.00  ns

# arrival time:    10.32  ns

# slack:           -0.32  ns   <-- NEGATIVE = fail

At the bottom of the report, OpenSTA prints WNS (worst negative slack). WNS ≥ 0 means you've closed setup.

## Step 6 · Timing closure technique 1 — tighten SDC

Before anything else, make sure your SDC is not LYING. Common lie: input delay too low. Real-world inputs arrive with ~2 ns of external setup — model it:

set_input_delay -clock clk -max 2.0 [get_ports {ui_in[*] uio_in[*]}]

set_output_delay -clock clk -max 2.0 [get_ports {uo_out[*] uio_out[*]}]

Tightening the SDC does NOT make timing worse — it makes it HONEST. If WNS was +0.5 ns under a loose SDC but -1.5 ns under a tight one, you never had 0.5 ns to begin with.

## Step 7 · Technique 2 — Restructure the critical path

Open the sta-worst.rpt and look at the path. If it's 15+ cells deep, you probably have a long combinational function — break it:

// BEFORE — one cycle, long path

always @(posedge clk)

result <= (a & b) | (c & d) | (e & f) | (g & h) | (i & j) | (k & l);

// AFTER — two cycles, balanced path

reg stage1_ab, stage1_cd, stage1_ef, stage1_gh, stage1_ij, stage1_kl;

always @(posedge clk) begin

stage1_ab <= a & b; stage1_cd <= c & d; stage1_ef <= e & f;

stage1_gh <= g & h; stage1_ij <= i & j; stage1_kl <= k & l;

result    <= stage1_ab | stage1_cd | stage1_ef

| stage1_gh | stage1_ij | stage1_kl;

end

E8873C

Adding a pipeline stage changes latency. Make sure the rest of your design tolerates it — the integration smoke test in test/top/ will catch most violations.

## Step 8 · Technique 3 — Upsize flop drive

Some flops have multiple drive strengths. Yosys picks the smallest by default; if the path is dominated by the flop-to-flop wire, upsizing the source flop from `_1` to `_4` can pay off.

# In config.json

"SYNTH_STRATEGY": "DELAY 4",

"SYNTH_BUFFERING": 1,

"SYNTH_SIZING": 1

## Step 9 · Technique 4 — Bump CLOCK_PERIOD (last resort)

If all else fails, raise CLOCK_PERIOD from 10 ns to 12 ns (83 MHz → 100 MHz). You lose the 100 MHz badge but you ship the chip.

# config.json

"CLOCK_PERIOD": 12.0

Grader rules: 10 ns earns full points; 12 ns is -50 XP; 15 ns is -150 XP; anything slower is a fail.

## Step 10 · Close hold

Hold failures are less common but much worse — they cannot be fixed by slowing the clock. Symptom in sta-worst.rpt:

Path Type: min (hold)

slack: -0.08 ns  <-- hold violation

Fix: add buffers on the offending path. LibreLane does this automatically if you set:

"RSZ_HOLD_MAX_BUFFER_PERCENT": 50

If the post-run report still shows hold fails, your reset synchroniser is probably the culprit — it has a tight hold window by design. Widen it to 3-flop stages.

## Troubleshooting

| LVS MATCH but timing fails by 0.01 ns | Tighten SDC a bit more then re-run. 10 ps of slack is real but bootstrappable. |
| --- | --- |
| LVS fails AFTER timing was clean | You changed a port name during closure. Update pin_order.cfg. |
| Hold fix breaks setup | RSZ_HOLD_MAX_BUFFER_PERCENT is too aggressive. Step down from 50 → 30. |
| OpenSTA crashes | Usually a malformed SDC. Run `sta -no_init` and source your sdc manually to see the parse error. |

## Reflection

- Record WNS-before and WNS-after for each technique you applied. This is a rich escape-log entry.

- Which technique gave you the biggest bang for the buck? Most learners: restructure (Step 7) > SDC tighten (Step 6) > upsize (Step 8) > period bump (Step 9).