# SILICON DREAMS

Study Guide

MODULE 3  ·  WEEK 3  ·  CM-HW-101-M3

SG-M3-08    pairs with TB-M3-08

# Launch Day: Tape-out Checklist and Reflection

Run the final sign-off, submit to the ChipFoundry shuttle, prepare for 12 weeks of silence.

## Learning objectives

- Execute the course-graded pre-submission checklist from end to end.

- Submit to the ChipFoundry shuttle portal without tripping any of the four rejection triggers.

- Set yourself up for a successful post-silicon bring-up in week 12.

- Write the Module 3 reflection for 300 XP.

## Step 1 · The pre-submission checklist

- DRC: zero errors under SKY130A. `runs/final/reports/signoff/drc.rpt` empty of ERROR lines.

- LVS: MATCH on both Magic and Netgen. `runs/final/reports/signoff/lvs.rpt` ends with 'Circuits match: MATCH'.

- Timing: WNS ≥ 0 worst corner, WHS ≥ 0 worst corner. Both `sta-worst.rpt` slacks positive.

- Antenna: zero violations. `reports/signoff/antenna.rpt` empty of ERROR.

- LibreLane version: 2025.04 confirmed in `metrics.csv`.

- Tag: `v1.0.0-final` pushed, CI green on all three workflows (drc.yml, lvs.yml, timing.yml).

- info.yaml: fields `author` and `discord` filled in — do this LAST, right before submission.

- escape-log.md: at least one entry per module (M1, M2, M3), reflection prompts answered.

- M2 rubric: score ≥ 600 recorded. M3 eligibility is gated on M2 completion.

- Integration smoke test: all 6 tests pass (from SG-M3-02).

- Arbiter fairness: elev_share ≥ 0.02 under priority adversary (from SG-M3-03).

- AXIOM shim: all three misbehaviour tests green (from SG-M3-05).

## Step 2 · Submit to the ChipFoundry shuttle

Update info.yaml with Your Real Information

# Update with your actual information

cat > info.yaml << 'EOF'

author: "Your Full Name"

discord: "YourDiscordHandle"

title: "Elevator Controller with Clock Gating and AXIOM Defensive Wrapper"

description: "Module 3 capstone: Elevator FSM with clock gating, AXIOM defensive wrapper, power optimization, and silicon verification"

EOF

# Or manually edit

nano info.yaml

Final Submission

Now everything should be ready! The checklist should show 10/10 passed.

# Run final checklist

./scripts/pre_submission_checklist.sh

# Build submission tarball

./scripts/build-submission.sh

# Complete submission

./scripts/complete_submission.sh

# This produces:

#   submission/silicon-dreams-<your-discord>-m3.tar.gz

#   Contents:

#     - runs/final/final/gds/top.gds

#     - runs/final/final/lef/top.lef

#     - runs/final/metrics.csv

#     - info.yaml

#     - COURSE_HASH (sha256 of your repo at tag v1.0.0-final)

# Upload via gh CLI (preferred)

gh release create v1.0.0-final submission/silicon-dreams-<your-discord>-m3.tar.gz \

--title "M3 tape-out submission" \

--notes "Silicon Dreams final submission"

# Or via the ChipFoundry portal

open https://portal.chipfoundry.io/silicon-dreams/submit

# Upload the tarball. Paste your GitHub release URL.

## Step 3 · The four rejection triggers

| Missing COURSE_HASH | Portal rejects silently. Always use build-submission.sh; don't hand-roll the tarball. |
| --- | --- |
| LibreLane != 2025.04 | Portal inspects metrics.csv. Any other version rejects. |
| info.yaml author blank | The course grader cross-references your Discord handle against the roster. Blank fails instantly. |
| GDS area > shuttle tile | Tile budget is 161 x 225 µm. If your GDS is larger, it physically cannot place. Check `metrics.csv → die_area`. |

## Step 4 · What happens after submission

The ChipFoundry grader runs a seven-stage post-submission flow on your tarball:

- COURSE_HASH verification (1 min)

- DRC re-run on their server (20 min)

- LVS re-run on their server (15 min)

- Timing re-run on their server (10 min)

- Rubric computation (1 min)

- Leaderboard update (1 min — only top 20 of the cohort shown)

- Merge into the shuttle multi-project wafer (manual; happens on next shuttle close date — could be up to 4 weeks)

You will receive an email within 1 hour with your final XP score. If rejected, you get 7 days to re-submit (one re-submission allowed).

## Step 5 · The 12-week wait

After the shuttle closes, fabrication takes ~10 weeks. Packaging + bring-up board shipping is another 2 weeks. During this time:

- Write up your design in `docs/writeup.md`. The course instructors will publish the best three as a cohort anthology.

- Prepare your bring-up plan — what's the first thing you'll toggle when the chip arrives? What does the oscilloscope trigger look like?

- Review the M3 textbooks once more. Chapter 8 has a bring-up checklist you'll actually use.

## Step 6 · Post-silicon bring-up (week 12)

You'll receive a bring-up board with your chip soldered down. The board exposes clk, rst_n, ui_in, uo_out, and uio via 0.1" headers. Your first job:

- Power the board from USB (5V, 500mA max). Confirm VDD=1.8V on the probe point.

- Apply a 10 MHz clock (NOT 100 MHz — start slow for bring-up). rst_n low for 100 cycles.

- Toggle ui_in[0] (request_strobe) with ui_in[4:1] = 4'h3. Watch uo_out[3:0] transition from 1 → 2 → 3.

- If the door_open bit (uo_out[7]) pulses after ~20 cycles, you've got silicon that works. Celebrate.

- Slowly bump the clock toward 100 MHz. Most of the cohort's chips work at 50-70 MHz in silicon even though STA reported 100 MHz — that's normal SKY130 slow-corner margin.

## Step 7 · The three chip failure modes

If your chip doesn't respond on day one, narrow it to one of three:

| Dead silicon | No response on any pin. Usually a power-rail short; the die was damaged in assembly. Report to ChipFoundry for replacement. Does not count against your rubric. |
| --- | --- |
| Partial function | Some outputs toggle, others don't. Most common pattern: arbiter grants visible on uio_out[2/3] but elevator state stuck. Check your reset synchroniser — it may have metastability at the lower clock. |
| Behavioural drift | Everything works but differently from simulation — off-by-one states, shifted counts. Almost always an SDC issue that snuck through. File it as a teaching moment in #m3-silicon. |

## Step 8 · The M3 reflection (300 XP)

In `notes/m3-reflection.md`, answer the six prompts below. Minimum 500 words total. The graders actually read these.

- What was the hardest single moment in Module 3, and what pulled you through it?

- If you had to do Module 3 over again, which decision would you change first?

- Which of the five AXIOM defensive rules will you take into every future project, and why?

- How did your mental model of timing change between SG-M3-01 and SG-M3-07?

- Write one paragraph to yourself on silicon bring-up day. What are you going to want to remember?

- What would you teach the Module 1 version of yourself, knowing what you know now?

## The final rubric

| Clean DRC/LVS/timing (final tag) | 500 XP |
| --- | --- |
| Arbiter fairness + 50 XP bonus | 250 XP |
| Clock gating + measurable power drop | 150 XP |
| AXIOM shim all three tests green | 250 XP |
| Reflection (500 words) | 300 XP |
| Bring-up success at week 12 | 150 XP |
| First-try shuttle accept | 100 XP bonus |
| TOTAL possible | 1700 XP |

## The last thing to do

Push the tag. Submit the tarball. Close the laptop lid. Module 3 is done. AXIOM saw you coming — the shim held. The shuttle closes in 3 days. Your silicon will arrive. We'll see you on bring-up day.

E8873C

You built a chip. An actual chip, on an actual wafer, that actual photons will hit when you plug in the bring-up board. Whatever grade this earns, that's the accomplishment. Silicon Dreams, cohort 2026-spring, signing off.